iNDUSTREAL

COMPLEX  IN  MOTION

DATASHEET

iNDUSTREAL.VCU

VEHICLE CONTROL UNIT — NVIDIA Jetson AGX Orin

| Datasheet | IR.VCU-V0.7 |
| --- | --- |
| Type | iREAL.VCU.ORIN |
| Board | industREAL.VCU.V1 |

INTRODUCTION

The iNDUSTREAL.VCU is a ruggedized vehicle control unit built around the NVIDIA Jetson AGX Orin module for mission-critical edge-AI computing, autonomy and real-time vehicle control. It pairs the Orin's GPU/DLA AI compute with a dedicated STM32H5 safety co-processor, a hardware TPM 2.0 root of trust, and automotive I/O set.

It provides dual CAN-FD, dual RS-485, an on-board 6-port Gigabit Ethernet switch, MIPI-CSI / GMSL2 camera input, USB 3.0, HDMI, and protected analog and digital I/O on a single automotive connector. A wide-range power front-end with load-dump and reverse-polarity protection (ISO 7637 / LV 124) keeps the unit alive through harsh vehicle transients. The system ships with a full Linux BSP (NVIDIA JetPack / L4T) with board-specific device tree, pinmux and flashing configuration.

APPLICATION EXAMPLES

- Automotive / Off-highway: autonomous vehicle controller, ADAS sensor fusion

- Robotics: mobile-robot and AMR central compute with vision

- Industrial: machine vision, edge-AI inference, data logging

- Defense / Transportation: rugged edge compute with camera and bus connectivity

COMPUTE UNIT

| Module | NVIDIA Jetson AGX Orin SOM (P3701) |
| --- | --- |
| Variants | 32 GB (P3701-0004) / 64 GB (P3701-0005); Industrial option |
| Memory | 256-bit LPDDR5x (32 GB / 64 GB) |
| GPU | NVIDIA Ampere, up to 2048 CUDA cores + 64 Tensor cores |
| CPU | Up to 12-core Arm Cortex-A78AE |
| AI performance | Up to 275 TOPS (64 GB module) |
| DL accelerators | 2× NVDLA |
| Operating system | Linux for Tegra (Ubuntu 20.04 / 22.04), JetPack 5.x / 6.x |

SAFETY & SECURITY CO-PROCESSORS

| Block | Device | Interface | Function |
| --- | --- | --- | --- |
| Safety MCU | STM32H523VE (Cortex-M33, 250 MHz, 512 KB Flash, 272 KB RAM) | UART4 + USB | Supervision, power sequencing, watchdog, real-time I/O, in-system DFU |
| Security | Infineon SLB9670 TPM 2.0 | SPI | Hardware root of trust / secure boot |
| Board temp | On-board sensor + TEMP_ALERT | I2C / GPIO | Thermal monitoring |

COMMUNICATION INTERFACES

| Interface | Qty | Details |
| --- | --- | --- |
| 1GBase-T Ethernet | 5× | On-board KSZ9896 6-port managed GbE switch (RGMII to Orin EQOS, SPI managed); rugged RJ45, 10/100/1000Base-T, AutoMDIX |
| CAN-FD | 2× | MCP2542FD transceivers, up to 8 Mb/s, 12 V/24 V systems; software standby; CAN/LIN TVS + common-mode choke; selectable termination via mounting option |
| RS-485 | 2× | Flexible-VIO transceivers, up to 20 Mb/s; ±30 kV ESD (IEC 61000-4-2/4/5) protected |
| USB 3.0 | 3× | Rugged Type-A, current-limited high-side switch, ESD-protected |
| HDMI | 1× | CEC / DDC / HPD, ESD-protected (for debugging purpose) |
| MIPI-CSI / GMSL2 camera | 4× | 120-pin high-speed B2B (14 Gbps); 3× camera MCLK, I2C, 4× camera interrupts; GMSL2 via BSP overlay |
| Debug console | 1× | UART (TX/RX) |

ANALOG & DIGITAL I/O

| I/O | Details |
| --- | --- |
| Analog inputs | Multiplexed (1:8 MUX) into quad rail-to-rail op-amps; TVS-protected front-end |
| Analog outputs | 4× (AN_OUT[0..3]) via quad 10-bit SPI DAC, op-amp buffered |
| Digital I/O | Protected GPIO with high-/low-side power-MOSFET drivers, current sense (0.15 Ω), resettable fuses, TVS clamps |
| PWM | General-purpose PWM output + fan PWM/tach |

All field I/O routed through the single automotive terminal connector.

POWER CHARACTERISTICS

| Input | Wide-range automotive DC, supports 12 V and 24 V systems |
| --- | --- |
| Transient protection | LM74810 ideal-diode controller — load-dump protection, ISO 7637 / LV 124, transient clamp to 65 V |
| Reverse polarity | Protected |
| Input fuse | 15 A fast (72 VDC rated) |
| Internal rails | VSYS, 12 V, 5 V, 3.3 V, 2.5 V, 1.8 V, 1.2 V |
| Auxiliary output | +5V (500mA PPTC-fused) for sensors VSYS (500m PPTC-fused) |
| Boost converter | TPS55330 (generates 12 V rail) |
| Operating input range | Contact iNDUSTREAL (application-dependent) |

MECHANICAL & ENVIRONMENTAL

| Main connector | MOLEX CMC automotive header, 48-pin, right-angle, sealed, -40 °C rated |
| --- | --- |
| Housing | Industrial / automotive enclosure |
| Cooling | Passive (reduced temp. range), Active (fan), temperature-monitored, throttling supported |
| Operating temperature | Per module: Commercial / Industrial (-40 … +85 °C with Industrial Orin) |
| Connectors | Rugged RJ45, USB 3.0 Type-A, HDMI (rated -40 … +105 °C) |
| Dimensions / weight | 265.75 x 183.9 x 49.2 (WxHxD), |

SOFTWARE SUPPORT

Ships with a complete Linux BSP and board support package for PCBA-FSI-VCU-01:

- Tested NVIDIA JetPacks:

        - JetPack 6.2.1/Jetson Linux 36.4.4

            - JetPack 7.2/Jetson Linux 39.2

- Board-specific MB1 BCT pinmux, GPIO and pad-voltage device trees

- Flashing configuration (fsi-vcu-01.conf) with EEPROM-less board ID bypass

- Reference examples: dual CAN-FD, dual RS-485, Ethernet switch management, STM32 MCU communication, MIPI-CSI / GMSL2 camera, analog/digital I/O

- AI model deployment via Orin GPU / DLA

GitHub URL: TBD

ORDER CODE

iR.VCU.<MEM>

| Code | Meaning |
| --- | --- |
| 32 | Jetson AGX Orin 32 GB (P3701-0004) |
| 64 | Jetson AGX Orin 64 GB (P3701-0005) |
| 64I | Jetson AGX Orin Industrial 64 GB |

All data for information purposes only. Subject to change without notice. © iNDUSTREAL